TECHNICAL FAULT DIAGNOSIS REPORT

Kausar Trade & Distributors — CNC Operations

| Report No. | MR-2026-014 | Date | 22 Aug 2026 |
| --- | --- | --- | --- |
| Machine ID | CNC-102 | Technician | M. Asad Abbas |

# 1. Issue Reported

Operator reported abnormal spindle temperature rise during extended operation on CNC-102, accompanied by intermittent vibration at higher RPM settings.

# 2. Inspection Findings

• Coolant flow rate found below specification due to partial blockage in the supply line.

• Spindle bearing housing showed mild heat discoloration, consistent with reduced coolant circulation.

• No abnormal wear detected on drive belts or tool holder assembly.

# 3. Corrective Action Taken

Coolant line was flushed and the filter element replaced. Flow rate was verified against manufacturer specification using a calibrated flow meter. Machine was run for 45 minutes under load to confirm temperature stabilized within the normal operating range.

# 4. Outcome

Spindle temperature returned to normal operating range (verified below 55°C at full load). Vibration no longer present. Machine returned to production.

# 5. Recommendation

Add coolant line inspection to the monthly preventive maintenance checklist to avoid recurrence.

Prepared by: Muhammad Asad Abbas

Position: Technician, CNC Operations